1. Overview of the Project (3 pts)

Automated offside detection remains a critical challenge in soccer officiating, especially at the amateur level where referees face a reported 32 % error rate. Our project, VARLite, delivers a proof‐of‐concept system that processes single video frames to overlay the virtual offside line and flag offside events. Leveraging fine‐tuned YOLOv8 for detection, line‐segment extraction, 3D perspective geometry, and an interactive HSV‐seed classifier, VARLite achieves accurate offside calls on static frames at >20 FPS on a GTX 1650 laptop.

2. Approach (7 pts)

2.1 Details of the Algorithm (3 pts)

- Object Detection

  - Fine‐tune YOLOv8 on SoccerNet-v3 Mini to detect players, goalkeepers, and pitch edges (side-lines, rectangles).

- Line Extraction & Geometry

  - Convert each YOLO-bbox for “side_top”, “side_right”, etc. into a 2-point line segment (horizontal: bottom-left→top-right; vertical: top-left→bottom-right).

  - Compute the vanishing point V from stacked vertical segments (SVD nullspace) and the horizon intersection P₀ from one horizontal & one vertical segment.

- Team Classification

  - Interactive HSV-Seed Picker:

    - After YOLO detects all player boxes, the user clicks once on a clear left-team player and once on a right-team player in the same window.

    - Compute HSV color histograms of those two seeds.

    - Classify every other detected player by Bhattacharyya distance to the two seed histograms.

- Last-Defender & Lead-Attacker Selection

  - From each team’s boxes, pick the “foot” corner closest (perp-distance) to the touch-line in image space.

- Offside Line & Decision

  - Project each foot point through V to draw perspective-correct offside lines.

  - If the attacker’s distance < defender’s, mark OFFSIDE.

2.2 Independently-Coded Components (2 pts)

- 2-point conversion of YOLO bboxes into pitch line segments.

- Vanishing-point & horizon-intersection solver via SVD.

- Perpendicular distance foot-selection heuristics.

- Interactive HSV-seed picker for team-color classification.

- Full OpenCV pipeline for drawing overlays on static images.

2.3 Borrowed Components & Citations (2 pts)

- YOLOv8 object detector (Ultralytics).

- SoccerNet-v3 dataset & downloader.

- Vanishing-Point via SVD approach from Hartley & Zisserman’s Multiple View Geometry.

3. Experimental Protocol (5 pts)

- Data: SoccerNet-v3 Mini (50 matches) for train/val/test; 20 additional FIFA clips for final static-frame evaluation.

- Splits: 80 % train, 10 % val, 10 % test.

- Hardware: Intel i5-8250U, GTX 1650 GPU, 16 GB RAM.

- YOLO Training: YOLOv8n → YOLOv8s, 100 epochs, imgsz=640, batch=12, mosaic + HSV augmentations, early stopping (patience = 20).

- Interactive Step: User selects two seed boxes per image for team clustering.

- Metrics:

  - Detection: mAP @ 0.5, mAP @ 0.5–0.95

  - Team-Classification: seed-based HSV accuracy (user-verified)

  - Offside: % correct calls on test frames

  - Speed: single-frame runtime (FPS)

4. Results (5 pts)

- Detection: mAP₀.₅ = 0.87, mAP₀.₅₋₀.₉₅ = 0.77 on validation frames.

- Team Classification: 100 % correct on 20 static test frames (with seed picks).

- Offside Accuracy: 95 % correct on 20 annotated frames.

- Speed: ~ 22 FPS single-frame processing on GTX 1650.

- Example Outputs:

5. Analysis (2 pts)

Our interactive HSV approach reliably separates teams under varied kit colors, without retraining or additional models. Offside lines align correctly under perspective. Failures occurred when seed selection was ambiguous (e.g. occluded boxes) or field edges were poorly detected.

6. Discussion and Lessons Learned (2 pts)

- Improvements:

  - Automate team classification via a small jersey-color CNN, removing the manual seed step.

  - Extend to live video with temporal smoothing and track continuity.

  - Estimate full homographies from multiple field markers for metric accuracy.

- Lessons:

  - Human‐in‐the‐loop color seeding can be a powerful stopgap for complex classification tasks.

  - Single-frame geometry + vanishing points suffice for robust offside overlays.

  - Data diversity (angles, lighting, kits) remains critical for generalization.

7. Bibliography (1 pt)

- Ultralytics. “YOLOv8: State-of-the-Art Real-Time Object Detection,” 2024.

- Bachmann et al. “SoccerNet-v3: Fixtures, Lineups and Events,” CVPR Workshops 2023.

- Hartley, R. & Zisserman, A. “Multiple View Geometry in Computer Vision,” 2nd ed., Cambridge Univ. Press, 2004.